# PF corrector requirements for HS and WFMOS

17th March 2006

Revised 2Oct07 based on Initial Baseline WFC Design for HyperSuprime

| Item no. | Description and notes | HS requirements | ## WFMOS joint requirements |
| --- | --- | --- | --- |
| 1 | Field of view. (With the mechanical restrictions imposed by the existing or only slightly modified Subaru prime focus hub, extension beyond 1.5º diameter is impractical and very unlikely at this point.) | 1.5º diameter | 1.5 degree diameter. Smaller field of view will be considered as part of the conceptual design study trade studies. |
| 2 | Range of acceptable focal ratio or effective focal length. | Focal length from 18 to 20.6 m. | As for HS with preference for upper end of focal length range |
| 3 | Spectral range to be covered with a single setting of telescope focus | Each of SDSS spectral windows g (398 to 522 nm), r (562 to 698 nm), I (706 to 850 nm), z (850 to 990 nm) and Y (~ 900 to 1000 nm). | 390-1000 nm. But cost of extension so far to blue should be assessed as part of optical design study. |
| 4 | Image quality, expressed as diameters (arcsec) enclosing 80%, 90%, and 95% of energy, by geometric optics for star at zenith. | For windows r, i, z: 80% within 0.3 arcsec. For windows g and Y: 80% within 0.5 and 0.4 arcsec, respectively. | Goal: 80% EE < 0.4”, 90% EE < 0.48”, 95% EE < 0.55” Requirement: 80% EE < 0.45”, 90% EE < 0.53”, 95% EE <0.60”. |
| 5 | Any relaxation of image quality requirement at wavelength extremes? | As above. | Image quality can be relaxed by factor of 1.12 at 390 nm in comparison to 700 nm image. Image quality at 1.0 micron should be no worse than at 700 nm and should have a goal of being better by up to a factor of 1.08 (i.e. smaller image by 0.92 times that at 700 nm). |
| 6 | Atmospheric dispersion correction. Range of zenith distance for which correction is required. | Full correction to 60º, usable to 65º . | 0 to 60 degrees ZD |
| 7 | Relaxed image quality requirement at large zenith distances. | At 45º, degradation by 10%; at 65º, degradation by 15% . | At ZD = 60 deg, the image quality may be degraded by factor of 1.5. The rate of degradation as a function of airmass should be no worse than the airmass to the 0.6 power |
| 8 | Minimum radius of curvature of focal surface. (Specify whether convex or concave to M1 or both are acceptable.) Is a flat field especially desirable? | 40 metre, either convex or concave. Flat field preferred. | Flat field is preferred but not required. Radius not to be less than ~10 meters, either convex or concave. |
| 9 | “Telecentricity” . This is hard to specify succinctly. What matters in a case such as feeding fibres set with their axes perpendicular to the focal surface is by how much the greatest inclination to the focal surface of any ray for any point in the field exceeds that for a marginal ray for a point on axis. For the prime focus correctors suiting the existing Subaru top end, there is off axis vignetting which considerably reduces the maximum inclination of the rays for images at the field periphery. | Not required. | Not required. Expected non-telecentricity may increase lost light at edge of field another 10% if fibers are oriented perpendicular to the focal plane and have entrance cones comparable to that of the f/2 primary mirror. |
| 10 | Vignetting. Maximum acceptable percentage vignetting at field periphery. With Subaru Option 1, vignetting of at least 25% is probably unavoidable at 0.75º radius. Vignetting increases almost linearly from the field centre to 0.75º radius and much more quickly beyond that, because of restriction on the diameter of rearward elements. | 25% aim. 28% worst case. | Maximum of ~30% at edge of 1.5 degree field, varying roughly linearly from the center. Some trades between maximum realized vignetting and effective field size should be made as part of the conceptual design study. |
| 11 | Corrector lens transmission (not including vignetting). Further corrector optimization work is needed to minimize the loss resulting from the 20 air/glass interfaces of the current baseline WFC design. | Minimum 75% from 500 to 1000 nm. | >80% at 390 and 1000 nm with >85% at 560. This requirement may be difficult to meet for 20 air/glass interfaces. |